# Optimizing Plane Design Using CFD Analysis

A parametric CFD study of CDF-62, a twin-jet transport-aircraft concept modelled at 1:100 scale. Three wing shapes, three fuselage shapes and three tail layouts were simulated one change at a time against a baseline, and the best option from each group was combined into a final design.

All eight configurations (1:100 model)

# Result

The final design, W2F2T3 (27° wing sweep, ogive nose, V-tail), improved on the baseline in every headline metric:

| Metric | Baseline W1F1T1 | Final W2F2T3 | Change |
| --- | --- | --- | --- |
| Maximum L/D | 9.0 | 9.7 | +7.8% |
| Minimum CD | 0.0341 | 0.0334 | −2.1% |
| Lift-curve slope CLα (per rad) | 4.08 | 4.24 | +3.9% |
| Pitch stability CMα (per rad) | −0.65 | −0.85 | 31% more stable |

Final design W2F2T3: perspective, top, side and front views

# Design variables

Each configuration is named W#F#T# (wing, fuselage, tail). The baseline is W1F1T1.

| Code | Wing (W) | Fuselage (F) | Tail (T) |
| --- | --- | --- | --- |
| 1 | 35° leading-edge sweep | Elliptical nose, 1.8 m tail upsweep | Conventional (fin + low tailplane) |
| 2 | 27° leading-edge sweep | Tangent-ogive nose, 13 m long | T-tail |
| 3 | 35° sweep + blended winglet | High tail upsweep (3.2 m), later tail-cone start | V-tail (38.6° dihedral) |

Common geometry (full scale): 62 m fuselage, 6.2 m diameter, 58.76 m projected span, low wing with 5° dihedral, NASA SC(2) supercritical airfoils (14% root to 10% tip), NACA 0010 tail surfaces. Projected span and mean-chord position are held constant across wing variants so comparisons are like-for-like.

# All configurations

Coarse mesh, U = 20 m/s, angle of attack −4° to 14°.

| Config | Change from baseline | Max L/D | Min CD | CLα (/rad) | CMα (/rad) |
| --- | --- | --- | --- | --- | --- |
| W1F1T1 | Baseline | 9.0 | 0.0341 | 4.08 | −0.65 |
| W2F1T1 | 27° sweep | 9.3 | 0.0348 | 4.18 | −0.57 |
| W3F1T1 | Winglet | 8.9 | 0.0341 | 3.94 | −0.54 |
| W1F2T1 | Ogive nose | 9.1 | 0.0336 | 4.09 | −0.70 |
| W1F3T1 | High upsweep | 8.6 | 0.0362 | 4.06 | −0.61 |
| W1F1T2 | T-tail | 8.9 | 0.0350 | 4.16 | −1.15 |
| W1F1T3 | V-tail | 9.3 | 0.0331 | 4.16 | −0.87 |
| W2F2T3 | Combined | 9.7 | 0.0334 | 4.24 | −0.85 |

Summary of all configurations

## Findings by group

- Wing. Reducing sweep to 27° gave the best L/D and lift slope. The winglet (W3) did not pay off at this Reynolds number: same minimum drag, lower lift slope.

- Fuselage. The ogive nose cut minimum drag and slightly improved stability. The high-upsweep tail cone added the most drag of any variant.

- Tail. The T-tail was by far the most stable (CMα −1.15) but cost L/D. The V-tail had the lowest drag of the whole study while still being much more stable than the baseline, so it was picked for the final design.

## Combined design

Combined study polars: baseline, V-tail only, and final design

The combined design has the highest L/D peak (around 4° AoA) and the steepest stable pitching-moment slope. One trade-off to note: above about 12° its lift curve flattens and drops earlier than the baseline, so its stall margin is smaller.

Polars for each group are in the plots folder (compare_wing.png, compare_fuselage.png, compare_tail.png). Pitching moment is taken about x = 336.8 mm on the 1:100 model.

# Repository layout

models/scad/   Parametric OpenSCAD source, one file per configuration

models/stl/    Exported 1:100 meshes (millimetres)

plots/         Polars and summary charts from the CFD runs

docs/images/   Renders of the configurations

# Using the models

The .scad files are all the same parametric model, each pre-set to one configuration with three switches at the top:

wing_type = 2;   // 1 = 35° sweep, 2 = 27° sweep, 3 = winglet

fuse_type = 2;   // 1 = baseline, 2 = ogive nose, 3 = high upsweep

tail_type = 3;   // 1 = conventional, 2 = T-tail, 3 = V-tail

Other settings:

- units = 10 exports millimetres of the 1:100 model (for printing); units = 0.01 exports metres of the 1:100 model (for CFD meshing).

- part selects "assembly" or a printable part: "fuselage_shell", "wing_right", "wing_left", "tail", or "cutaway" (preview with internal components).

- te_min sets a minimum trailing-edge thickness. Keep 0 for CFD; use 0.05 (0.5 mm on the print) for FDM printing.

Open in OpenSCAD 2021.01 or newer, render with F6, then File > Export > STL. Fusion 360 can't open .scad files directly, so import the STL instead.

The printable layout is sized for a 1:100 flying model: 1.2 mm skin, a 6 mm carbon spar tube from root to kink, a top hatch, and space for a 3S 850 mAh LiPo, 30 A ESC, receiver, tail servo and 40 mm EDF.

# Limitations

- All results are from a coarse mesh; a mesh-independence study is still needed before quoting absolute values.

- Runs are at U = 20 m/s on a 1:100 model, so the Reynolds number is far below full scale. Trends between configurations are more reliable than absolute coefficients.

- Each group was varied with the other two held at baseline; interaction effects were only checked for the one combined design.